## PAPER TITLE (Capitalize the first letter of all major words)

Considerations for Prioritization in the Management of a Portfolio of Dams

Relevant Topic: (Highlight Selected Topic in Bold font)

- Water Planning, Water Management, and Climate Resilience

- Dam Safety Policy and Governance

- Dam Construction and Rehabilitation: Innovation and Lifecycle Extension

- Dam Performance Monitoring

- Flood Resiliency in Developed and Developing Countries

- Sedimentation Management and Reservoir Longevity

- Fish Passage, Biodiversity & Environmental Integration

- Community Engagement in Dam Development

- Tailings Dam Safety

- Dam Decommissioning & Removal

AUTHORS

Primary Contact Author (Author Full Name, Company Affiliation/Employer, Address, Phone, email

David S. Bowles, Managing Principal, RAC Engineers and Economists, LLC and Professor Emeritus of Civil and Environmental Engineering, Utah State University.  735 E Ave., Coronado, CA 92118, U.S.A.  +1.435.770.8709  David_S_Bowles@hotmail.com

Is Primary Author a Young Professional? YES / NO (Highlight YES in Bold font if under 40 years old)

Co-Author Information (Author Full Name, Company Affiliation/Employer, Address, Phone, email

Co-Author Information (Author Full Name, Company Affiliation/Employer, Address, Phone, email

Co-Author Information (Author Full Name, Company Affiliation/Employer, Address, Phone, email

ABSTRACT: Abstract shall be no more than 300 words and cannot include figures, tables, drawings, references, or equations. Abstracts should provide a brief overview of the paper, highlighting relevance to the selected topic, key findings/conclusions, and significance to the industry.  Provide Spanish translation of abstract in the provided space to aid in review and selection.

PUBLISHED PAPER

or

PRESENTATION ONLY

(Highlight Choice of Published Paper or Presentation Only in Bold font)

ENGLISH VERSION OF ABTRACT:

Prioritization and associated funding queues are necessary due to resource limitations and to reduce overall portfolio risk efficiently.  In addition, uncertainty should be reduced through investigations and analyses.  Examples of queues include: Issue Evaluation studies; operation and maintenance recommendations; risk reduction options analysis and conceptual designs; and final design and construction of risk reduction measures (RRMs).  Each queue contains the set of dams or RRMs awaiting studies or processing to the next step in the overall portfolio risk management process.  While the intent is that each queue is eventually cleared, it is possible that a dam could come into a queue and move ahead of others that are already in the queue.  There are no queues for expedited actions since funding must be found to implement them immediately to prevent an imminent dam failure.

The paper describes an integrated framework for prioritization in portfolio risk management, which has been implemented on portfolios of varying sizes.  The framework is dynamic, with changes to priority being made as better information becomes available or other changes take place.  An important distinction is made between “priority” (the order in which things should be done) and “urgency” (how soon things should be done).

Alternative prioritization criteria, such as “worst first” or cost effectiveness of risk reduction, are evaluated.  The paper also discusses achieving a faster rate of risk and uncertainty reduction through staging risk reduction and staging investigations and analyses, respectively. Practical examples are drawn from advising owners and regulators in many countries with portfolio risk management.  These include: the role of interim RRMs; grouping of dam safety issues for investigations or implementation of RRMs at multiple dams; systems considerations (dams in a cascade, risk trading); coordination of dam safety work with capacity expansion; and transitioning to the use of a risk-informed approach.

SPANISH TRANSLATION OF ABSTRACT:

La priorización y las colas de financiamiento asociadas son necesarias debido a las limitaciones de recursos y para reducir de manera eficiente el riesgo general de la cartera. Además, la incertidumbre debe reducirse mediante investigaciones y análisis. Algunos ejemplos de colas incluyen: estudios de evaluación de problemas; recomendaciones de operación y mantenimiento; análisis de opciones de reducción de riesgos y diseños conceptuales; y diseño final y construcción de medidas de reducción de riesgo (MRR). Cada cola contiene el conjunto de presas o MRR que esperan estudios o el procesamiento al siguiente paso en el proceso general de gestión de riesgo de la cartera. Aunque la intención es que eventualmente se despeje cada cola, es posible que una presa ingrese a una cola y avance por delante de otras que ya están en ella. No existen colas para acciones expedidas, ya que se debe encontrar financiamiento para implementarlas de inmediato y prevenir una falla inminente de la presa.

El artículo describe un marco integrado para la priorización en la gestión de riesgos de cartera, que se ha implementado en carteras de diferentes tamaños. El marco es dinámico, permitiendo que los cambios en la prioridad se realicen a medida que se obtiene mejor información o se producen otros cambios. Se realiza una distinción importante entre "prioridad" (el orden en el que deben realizarse las cosas) y "urgencia" (qué tan pronto deberían realizarse las cosas).

Se evalúan criterios alternativos de priorización, como “peor primero” o la rentabilidad de la reducción de riesgos. El artículo también analiza cómo lograr una tasa más rápida de reducción de riesgos e incertidumbres mediante la reducción de riesgos por etapas y la realización de investigaciones y análisis por etapas, respectivamente. Se presentan ejemplos prácticos basados en la asesoría a propietarios y reguladores en numerosos países con gestión de riesgos de cartera. Estos incluyen: el papel de los RRMs provisionales; la agrupación de problemas de seguridad de presas para investigaciones o la implementación de RRMs en múltiples presas; consideraciones sistémicas (presas en cascada, intercambio de riesgos); coordinación del trabajo de seguridad de presas con la expansión de capacidad; y la transición hacia el uso de un enfoque informado por riesgos.