PAPER TITLE (Capitalize the first letter of all major words)

Three-Dimensional Simulation Analysis of Oya Dam’s Behavior during the 2024 Noto Peninsula Earthquake in Japan and consideration of result

Three-Dimensional Simulation Analysis of Oya Dam’s Behavior during the 2024 Noto Peninsula Earthquake in Japan and consideration of result

Relevant Topic: (Highlight Selected Topic in Bold font)

- Water Planning, Water Management, and Climate Resilience

- Dam Safety Policy and Governance

- Dam Construction and Rehabilitation: Innovation and Lifecycle Extension

- Dam Performance Monitoring

- Flood Resiliency in Developed and Developing Countries

- Sedimentation Management and Reservoir Longevity

- Fish Passage, Biodiversity & Environmental Integration

- Community Engagement in Dam Development

- Tailings Dam Safety

- Dam Decommissioning & Removal

AUTHORS

Primary Contact Author (Author Full Name, Company Affiliation/Employer, Address, Phone, email

Kohei Kusumi, National Institute for Land and Infrastructure Management, 1 Asahi Tsukuba City Ibaraki prefecture Japan, +81-29-864-4597, kusumi-k924a@mlit.go.jp

Is Primary Author a Young Professional? YES / NO (Highlight YES in Bold font if under 40 years old)

Co-Author Information (Author Full Name, Company Affiliation/Employer, Address, Phone, email

Kenichi Kanenawa, National Institute for Land and Infrastructure Management, 1 Asahi Tsukuba City Ibaraki prefecture Japan, +81-29-864-4667, kanenawa-k92ta@mlit.go.jp

Kenichi Kanenawa, National Institute for Land and Infrastructure Management, 1 Asahi Tsukuba City Ibaraki prefecture Japan, +81-29-864-4667, kanenawa-k92ta@mlit.go.jp

Co-Author Information (Author Full Name, Company Affiliation/Employer, Address, Phone, email

Toshiyuki Sakurai, National Institute for Land and Infrastructure Management, 1 Asahi Tsukuba City Ibaraki prefecture Japan, +81-29-864-4667, sakurai-t92a@mlit.go.jp

Toshiyuki Sakurai, National Institute for Land and Infrastructure Management, 1 Asahi Tsukuba City Ibaraki prefecture Japan, +81-29-864-4667, sakurai-t92a@mlit.go.jp

Co-Author Information (Author Full Name, Company Affiliation/Employer, Address, Phone, email

Kenji Shimoyama, NEWJEC Inc., 2-3-20 Honjo-Higashi kita-ku Osaka city Osaka prefecture Japan, +81-6-6374-4315, shimoyamakn@newjec.co.jp

Co-Author Information (Author Full Name, Company Affiliation/Employer, Address, Phone, email

Shogo Takeda, NEWJEC Inc., 2-3-20 Honjo-Higashi kita-ku Osaka city Osaka prefecture Japan, +81-6-7636-8422, takedasg@newjec.co.jp

Shogo Takeda, NEWJEC Inc., 2-3-20 Honjo-Higashi kita-ku Osaka city Osaka prefecture Japan, +81-6-7636-8422, takedasg@newjec.co.jp

Co-Author Information (Author Full Name, Company Affiliation/Employer, Address, Phone, email

Ayumu Nakano, NEWJEC Inc., 2-3-20 Honjo-Higashi kita-ku Osaka city Osaka prefecture Japan, +81-6-6374-4597, nakanoym@newjec.co.jp

Ayumu Nakano, NEWJEC Inc., 2-3-20 Honjo-Higashi kita-ku Osaka city Osaka prefecture Japan, +81-6-6374-4597, nakanoym@newjec.co.jp

Co-Author Information (Author Full Name, Company Affiliation/Employer, Address, Phone, email

Hiroyuki Sato, Japan Dam Engineering Center, 2F Ikenohata Nisshoku Building 2-9-7 Ikenohata Taito-ku Tokyo Japan, +81-35-815-4161, h-satou@jdec.or.jp

Hiroyuki Sato, Japan Dam Engineering Center, 2F Ikenohata Nisshoku Building 2-9-7 Ikenohata Taito-ku Tokyo Japan, +81-35-815-4161, h-satou@jdec.or.jp

Co-Author Information (Author Full Name, Company Affiliation/Employer, Address, Phone, email

Yoshikazu Yamaguchi, Japan Dam Engineering Center, 2F Ikenohata Nisshoku Building 2-9-7 Ikenohata Taito-ku Tokyo Japan, +81-35-815-4161, yamaguchi@jdec.or.jp

Yoshikazu Yamaguchi, Japan Dam Engineering Center, 2F Ikenohata Nisshoku Building 2-9-7 Ikenohata Taito-ku Tokyo Japan, +81-35-815-4161, yamaguchi@jdec.or.jp

ABSTRACT: Abstract shall be no more than 300 words and cannot include figures, tables, drawings, references, or equations. Abstracts should provide a brief overview of the paper, highlighting relevance to the selected topic, key findings/conclusions, and significance to the industry.  Provide Spanish translation of abstract in the provided space to aid in review and selection.

PUBLISHED PAPER

or

PRESENTATION ONLY

(Highlight Choice of Published Paper or Presentation Only in Bold font)

ENGLISH VERSION OF ABTRACT:

During the 2024 Noto Peninsula Earthquake, magnitude Mw 7.6, which occurred on January 1, 2024, large seismic motion with a maximum acceleration of 528 cm/s2 in the stream direction was observed on the seismometer installed at the bottom inspection gallery of Oya Dam, (ER), epicentral distance: 12.2 km, with a height of 56.5meters, which is managed by Ishikawa Prefectural Government.

In this study, three-dimensional dynamic analysis was used to reproduce and analyze the behavior of the Oya Dam embankment during the earthquake, particularly focusing on the behavior along the dam axis, which cannot be adequately represented by two-dimensional analysis.

The nonlinearity of the embankment materials was taken into account using the equivalent linearization method.

As a result of the analysis, the response acceleration at the seismometer location of Oya Dam was in good agreement with that from the observation records, and the behavior of Oya Dam during the earthquake was reproduced well.

In addition, large accelerations, strains, and small tensile stresses were generated in the analysis at the locations where the deformation of the embankment was observed by the survey conducted immediately after the earthquake, and it is considered that the analysis can express the behavior of Oya Dam during the earthquake.

However, no results that would affect the stability of the embankment were confirmed in the analysis.

SPANISH TRANSLATION OF ABSTRACT:

Durante el terremoto de la península de Noto de 2024, magnitud 7,6, ocurrido el 1 de enero de 2024, se registró un fuerte movimiento sísmico con una aceleración máxima de 528 cm/s2 en la dirección horizontal en el sismógrafo instalado en la galería de inspección inferior de la presa de Oya (presa de enrocado con núcleo impermeable), con una altura de 56,5 metros, gestionada por el Gobierno de la prefectura de Ishikawa.

La distancia epicentral de la presa Oya es de 12,2 km.

En este estudio, se realizó un análisis dinámico tridimensional para simular y analizar el comportamiento de la presa de Oya durante el terremoto, centrándose especialmente en el comportamiento a lo largo del eje de la presa, que no puede representarse adecuadamente mediante un análisis bidimensional.

Se tuvo en cuenta la no linealidad de los materiales del dique mediante el método de linealización equivalente.

Los resultados del análisis mostraron una buena concordancia entre la aceleración registrada por el sismógrafo y los datos observados, lo que permitió reproducir con precisión el comportamiento de la presa durante el terremoto.

Además, en el análisis se generaron grandes aceleraciones, deformaciones y pequeñas tensiones de tracción en los lugares donde se observó la deformación del terraplén en el estudio realizado inmediatamente después del terremoto, y se considera que el análisis puede expresar el comportamiento de la presa de Oya durante el terremoto.

No obstante, el análisis no reveló ningún resultado que pudiera afectar la estabilidad del dique.

Keyword: Rockfill dam, Strong motion, Vibration characteristics, Equivalent linearization method, Three-dimentional finite element analysis

1-2 sentences on thematic relevance: Recent lessons from incidents and accidents concerning dams during the life cycle, including during construction.