## PAPER TITLE (Capitalize the first letter of all major words)

Enhancing Dam Safety through Dam Breach Analysis for Resilience to Climate Change

Relevant Topic: (Highlight Selected Topic in Bold font)

- Water Planning, Water Management, and Climate Resilience

- Dam Safety Policy and Governance

- Dam Construction and Rehabilitation: Innovation and Lifecycle Extension

- Dam Performance Monitoring

- Flood Resiliency in Developed and Developing Countries

- Sedimentation Management and Reservoir Longevity

- Fish Passage, Biodiversity & Environmental Integration

- Community Engagement in Dam Development

- Tailings Dam Safety

- Dam Decommissioning & Removal

AUTHORS

Primary Contact Author (Author Full Name, Company Affiliation/Employer, Address, Phone, email

Tumwine Nicholas1, Ministry of Water and Environment, P.O. Box 20026, Kampala, Uganda, +256770833347, tumwinen52@gmail.com / nicholas.tumwine@mwe.go.ug

Is Primary Author a Young Professional? YES / NO (Highlight YES in Bold font if under 40 years old)

Co-Author Information (Author Full Name, Company Affiliation/Employer, Address, Phone, email

Co-Author Information (Author Full Name, Company Affiliation/Employer, Address, Phone, email

Co-Author Information (Author Full Name, Company Affiliation/Employer, Address, Phone, email

ABSTRACT: Abstract shall be no more than 300 words and cannot include figures, tables, drawings, references, or equations. Abstracts should provide a brief overview of the paper, highlighting relevance to the selected topic, key findings/conclusions, and significance to the industry.  Provide Spanish translation of abstract in the provided space to aid in review and selection.

PUBLISHED PAPER

or

PRESENTATION ONLY

(Highlight Choice of Published Paper or Presentation Only in Bold font)

ENGLISH VERSION OF ABTRACT:

In Uganda, little is known about the possible extent of damage to infrastructure, reservoirs, lives, and environment in case of dam failure during extreme weather conditions. This study aimed at undertaking a dam breach analysis on Kibimba dam to find potential impacts due to extreme conditions such as flooding on lives, infrastructure, and basins. A digital terrain model was prepared from global satellite data using QGIS, and then imported to HEC-RAS hydraulic model which was setup with full reservoir storage of 13.5 Mm³, and an estimated flow of 100m3/s based on catchment characteristics during a large flood, equivalent to 1000 years return period. The dam was assumed to breach due to overtopping. Breach development in the hydraulic model was made relevant to dam material and initial natural state of the watercourse. Dam break was done with the full Saint Venants equations. Model results showed inundation of a large area approximately 830 hectares of rice paddy, 1200 workers were affected, 5 buildings inundated, 86 deaths, main highway bridge damaged, 1000m of the highway was damaged, 4950 cargo trucks affected, annual rice deficit of 50,000MT, annual government revenue from rice production of approximately USD 138,479. There were physical damages and changes to the environment due to erosion and sediment and altering of the wetland/basin environment. Model results revealed that in extreme conditions, consequences of dam failure will be extremely enormous. It is recommended that dam breach analysis be undertaken in all basins with dams and reservoirs to enable dam owners, communities, policy makers, and regulators to undertake planning early warning systems for dam safety, control of reservoirs, and basins from damage during extreme conditions. Future studies should ensure that flow gauges are functional and flood calculations are available to improve accuracy of model results.

Key words: (dam failure, flooding, communities)

SPANISH TRANSLATION OF ABSTRACT:

En Uganda, se sabe poco sobre el posible alcance de los daños que podrían sufrir las infraestructuras, los embalses, las vidas y el medio ambiente en caso de rotura de una presa en condiciones meteorológicas extremas. El objetivo de este estudio era realizar un análisis de la rotura de la presa de Kibimba para determinar los posibles impactos de condiciones extremas, como las inundaciones, sobre las vidas, las infraestructuras y las cuencas. Se preparó un modelo digital del terreno a partir de datos satelitales globales utilizando QGIS, y luego se importó al modelo hidráulico HEC-RAS, que se configuró con un almacenamiento total del embalse de 13,5 Mm³ y un caudal estimado de 100 m3/s basado en las características de la cuenca hidrográfica durante una gran inundación, equivalente a un período de retorno de 1000 años. Se supuso que la presa se rompería debido al desbordamiento. El desarrollo de la rotura en el modelo hidráulico se hizo relevante para el material de la presa y el estado natural inicial del curso de agua. La rotura de la presa se realizó con las ecuaciones completas de Saint Venants. Los resultados del modelo mostraron la inundación de una gran superficie de aproximadamente 830 hectáreas de arrozales, 1200 trabajadores afectados, 5 edificios inundados, 86 muertos, daños en el puente de la carretera principal, daños en 1000 m de la carretera, 4950 camiones de carga afectados, un déficit anual de arroz de 50 000 toneladas métricas y unos ingresos anuales del gobierno por la producción de arroz de aproximadamente 138 479 dólares estadounidenses. Se produjeron daños físicos y cambios en el medio ambiente debido a la erosión y los sedimentos, así como a la alteración del entorno de los humedales y las cuencas. Los resultados del modelo revelaron que, en condiciones extremas, las consecuencias de la rotura de la presa serían enormes. Se recomienda

Traducción realizada con la versión gratuita del traductor DeepL.com